# SM-3 Smart Electricity Meter - Datasheet

Meridian Infra Ltd - Metering Products

# Overview

The SM-3 is a single-phase smart electricity meter for homes and small shops. It measures energy in both directions, so it supports rooftop solar, and it sends readings to the head-end system every 15 minutes.

# Measurement

Accuracy class 1.0 for active energy (IS 16444). Rated current 5-60 A, 240 V, 50 Hz. Records voltage, current, power factor and maximum demand.

# Communication

NB-IoT cellular module with automatic fallback to RF mesh. Optical port for local reading. Firmware can be upgraded over the air.

# Tamper protection

The meter raises an event when the cover is opened, when a strong magnet is placed near it, when the neutral wire is disconnected or when current flows in reverse unexpectedly. Each event is time-stamped, stored in memory and sent to the head-end immediately.

# Prepaid mode

In prepaid mode the meter keeps a credit balance, warns the customer by SMS when credit is low and disconnects the supply through its built-in relay when credit runs out, except on public holidays and at night.

# Environment and warranty

Operating temperature -10 to 55 degrees Celsius, IP54 enclosure, flame-retardant polycarbonate case. Warranty: five years from installation.